中国科学技术大学计算机学院

《数字电路实验》报告

实验题目：实验 07 FPGA 实验平台及 IP 核使用

学生姓名：Ouedraogo Ezekiel B.

学生学号：PL19215001

完成日期：12/06/2020

计算机实验教学中心制

2020年10月

【实验题目】

实验 07 FPGA 实验平台及 IP 核使用

【实验目的】

熟悉 FPGAOL 在线实验平台结构及使用

掌握 FPGA 开发各关键环节

学会使用 IP 核（知识产权核）

【实验环境】

VLAB 平台：vlab.ustc.edu.cn

FPGAOL 平台：fpgaol.ustc.edu.cn

Vivado

Logisim

【实验过程】

- 使用时钟管理单元 IP 核

  - 通过计数器产生一个低频的脉冲信号

Verilog 代码

module clk_10(

input clk,

input rst,

output reg led,

output reg [3:0]count,

output pulse

);

always@(posedge clk)

begin

if(rst || count >= 4'h9)

count <= 4'h0;

else

count <= count + 4'h1;

end

assign pulse = (count == 4'h1);

always@(posedge clk)

begin

if(rst)

led <= 1'b0;

else if(pulse)

led <= ~led;

end

endmodule

Verilog 仿真代码

module test_bunch(

);

reg clk,rst;

wire [3:0]count;

wire pulse, led;

clk_10 clk_10(.clk(clk),.rst(rst),.led(led),.count(count),.pulse(pulse));

initial clk = 1'b0;

always #5 clk = ~clk;

initial

begin

rst = 1'b1;

#6 rst = 1'b0;

#1000 $finish;

end

endmodule

其仿真波形如下图所示

  - 通过时钟管理单元 IP 核生成

用频率为100Mhz的时钟生成频率10Mhz和200Mhz时钟

仿真文件代码

module test(

input clk,

input rst,

output [7:0] led);

wire clk_10m,clk_200m,locked;

reg [31:0] cnt_1,cnt_2;

always@(posedge clk_200m)

begin

if(~locked)

cnt_1 <= 32'hAAAA_AAAA;

else

cnt_1 <= cnt_1+1'b1;;

end

always@(posedge clk_10m)

begin

if(~locked)

cnt_2 <= 32'hAAAA_AAAA;

else

cnt_2 <= cnt_2+1'b1;;

end

assign led = {cnt_1[27:24],cnt_2[27:24]};

clk_wiz_0 clk_wiz_0_inst(

.clk_in1 (clk),

.clk_out1 (clk_10m),

.clk_out2 (clk_200m),

.reset (rst),

.locked (locked));

endmodule

- 使用片内存储单元

Logism 中的ROM和RAM

ROM

RAM

利用Vivado中的存储器

例化一个16*3bit的RAM

初始化

仿真文件代码

module tb( );

reg [3:0] dpra;

wire [2:0] dpo;

reg clk;

reg we;

reg [3:0] a;

reg [2:0] d;

initial

begin

clk = 0;

forever

#5 clk = ~clk;

end

initial

begin

a = 0;dpra=0;d=0;we=0;

#20

repeat(5)

begin

@(posedge clk); #1;

dpra = dpra +1;

end

repeat(10)

begin

@(posedge clk); #1;

a = $random%16;

dpra = $random%16;

d = $random%8;

we = $random%2;

end

@(posedge clk); #1;

a = 0;

dpra = 0;

d = 0;

we = 0;

#20 $stop;

end

dist_mem_gen_0 dist_mem_gen_0(

.a (a),

.d (d),

.dpra (dpra),

.clk (clk),

.we (we),

.dpo (dpo));

endmodule

其仿真波形如下图

【实验练习】

- 例化一个 16*8bit 的 ROM

初始化

代码

module rom(

input [3:0]a,

output [3:0]out

);

wire [7:0]spo;

assign out = spo[3:0];

image_rom rom(.a(a),.spo(spo));

endmodule

XDC 文件

结果

- 式将开关的十六进制数值在两个数码管上显示

例化一个 16*4bit 的 ROM

初始化

例化10Mhz的时钟

Verilog 代码

module rom(

input clk,

input [7:0]sw,

output reg [3:0]out,

output reg [2:0]an

);

wire clk_out1;

wire [3:0]spo0,spo1;

image_rom rom0(.a(sw[3:0]),.spo(spo0));

image_rom rom1(.a(sw[7:4]),.spo(spo1));

clk_10m clk_10m(.clk_in1(clk),.reset(1'b0),.clk_out1(clk_out1));

always@(*)

begin

case(clk_out1)

0:  begin

an = 3'b0;

out = spo0;

end

1:  begin

an = 3'b1;

out = spo1;

end

endcase

end

endmodule

XDC文件

结果

- 设计一个精 度为 0.1 秒的计时器

Verilog代码

module display(

input clk,

input rst,

output reg [3:0] out,

output reg [2:0] an

);

//create 10mhz clk

wire clk_out1;

clk_10m clk_10m(

.clk_in1(clk),

.reset(rst),

.clk_out1(clk_out1)

);

//get time(min,sec,...)

wire [3:0]tenth,sec,ten_sec,min;

count_time count_time(

.clk_10m(clk_out1),

.rst(rst),.tenth(tenth),

.sec(sec),.ten_sec(ten_sec),

.min(min)

);

//display

reg [1:0]count;

always@(posedge clk_out1)

begin

if(rst) count <= 2'b0;

else count <= count + 2'b1;

end

always@(*)

begin

case(count)

2'h0:begin

an <= 3'h0;

out <= tenth;

end

2'h1:begin

an <= 3'h1;

out <= sec;

end

2'h2:begin

an <= 3'h2;

out <= ten_sec;

end

2'h3:begin

an <= 3'h3;

out <= min;

end

endcase

end

endmodule

module count_time(

input clk_10m,

input rst,

output reg [3:0] tenth, sec, ten_sec, min

);

//create 0.1s pulse signal

reg [19:0]count;

wire pulse;

always@(posedge clk_10m or posedge rst)

begin

if(rst || count >= 20'hf423f) //hex f 423f = dec 999 999

count <= 20'h0;

else

count <= count + 20'h1;

end

assign pulse = (count == 20'h1);

//count time

//tenth sec

always@(posedge clk_10m or posedge rst)

begin

if(rst) tenth <= 4'h4;

else if(pulse)

begin

if(tenth >= 4'h9) tenth <= 4'h0;

else              tenth <= tenth + 4'h1;

end

end

//sec

always@(posedge clk_10m or posedge rst)

begin

if(rst) sec <= 4'h3;

else if(pulse)

begin

if(sec >= 4'h9 && tenth >= 4'h9)

sec <= 4'h0;

else if(tenth >= 4'h9) sec <= sec + 4'h1;

end

end

//ten_sec

always@(posedge clk_10m or posedge rst)

begin

if(rst) ten_sec <= 4'h2;

else if(pulse)

begin

if(ten_sec >= 4'h5 && sec >= 4'h9 && tenth >= 4'h9)

ten_sec <= 4'h0;

else if(sec >= 4'h9 && tenth >= 4'h9) ten_sec <= ten_sec + 4'h1;

end

end

//min

always@(posedge clk_10m or posedge rst)

begin

if(rst) min <= 4'h1;

else if(pulse)

begin

if(min >= 4'h9 && ten_sec >= 4'h5 && sec >= 4'h9 && tenth >= 4'h9)

min <= 4'h0;

else if(ten_sec >= 4'h5 && sec >= 4'h9 && tenth >= 4'h9) min <= min + 4'h1;

end

end

endmodule

XDC文件

结果

【总结与思考】

通过本次实验我们熟悉了 FPGAOL 在线实验平台结构及使用，掌握了 FPGA 开发各关键环节也学会了如何使用 IP 核。