中国科学技术大学计算机学院

《数字电路实验》报告

实验题目：贪吃蛇游戏

学生姓名：Ouedraogo Ezekiel B.

学生学号：PL19215001

完成日期：1/12/2021

计算机实验教学中心制

2020年10月

【实验环境】

Vivado

FPGA板子

【实验过程】

首先设计游戏的背景

module cel_color(

input VGA_clk,

input displayArea,

input [10:0] xCount, yCount,

output reg cel, in_space, cel_border, out_space

);

parameter CEL_SIZE = 32;

parameter OUT_SPACE = 1;

parameter IN_SPACE = 2;

parameter BORDER = 2;

wire [10:0] q_x = xCount/(CEL_SIZE);

wire [10:0] q_y = yCount/(CEL_SIZE);

wire [7:0] cel_x = xCount - q_x*CEL_SIZE;

wire [7:0] cel_y = yCount - q_y*CEL_SIZE;

integer i = IN_SPACE + BORDER + OUT_SPACE;

integer j = BORDER + OUT_SPACE;

integer k = OUT_SPACE;

always@(posedge VGA_clk)

begin

if(displayArea)

begin

cel <= (cel_x >= i) && (cel_y >= i) && (cel_x < CEL_SIZE - i) && (cel_y < CEL_SIZE - i) ? 1 : 0;

if(cel)

begin

in_space   <= 0;

cel_border <= 0;

out_space  <= 0;

end

else

begin

in_space = (cel_x >= j) && (cel_y >= j) && (cel_x < CEL_SIZE - j) && (cel_y < CEL_SIZE - j) ? 1 : 0;

if(in_space)

begin

cel_border <= 0;

out_space  <= 0;

end

else

begin

cel_border = (cel_x >= k) && (cel_y >= k) && (cel_x < CEL_SIZE - k) && (cel_y < CEL_SIZE - k) ? 1 : 0;

if(cel_border)

out_space  <= 0;

else

out_space = 1;

end

end

end

end

endmodule

结果

分析：输入信号为VGA_clk，displayArea，xCount，yCount。其中displayArea = 1时，表示在显示区域。 xCount和yCount表示位置。

看结果图可以发现背景时很多小正方形组成的。每个正方形含四部分，如下图:

cel

in_space

out_space

cel_border

到每个变量指的部分，其输出变量为1否则为0。四部分都可以设置自己的颜色。所以用模块就可以显示背景，蛇，目标和墙， 只需要到每个背景，蛇，目标或墙时候换颜色。

其次给目标设计一个输出随机位置的模块

module randomGrid(

input VGA_clk,

input [10:0] max_X, min_X,

input [10:0] max_Y, min_Y,

output reg [10:0] rand_X,

output reg [10:0] rand_Y

);

always @(posedge VGA_clk)

begin

rand_X <= ((rand_X + 3) % (max_X-min_X)) + min_X;

rand_Y <= ((rand_Y + 5) % max_Y-min_Y) + min_Y;

end

endmodule

在设计一个VGA控制模块

module VGA_gen(

input   CLK, RESET, //clk=65MHzz

output displayArea,

output reg [10:0] xCount,yCount, //x,y pixel

output reg VGA_HS,VGA_VS

);

parameter H_CNT = 11'd1343; //136+160+1024+24=1344-1

parameter V_CNT = 11'd805; //6+29+768+3=806-1

reg h_de, v_de; //data enable

reg [10:0] h_cnt,v_cnt;

always@(posedge CLK)

begin

if(RESET)

h_cnt <= 11'd0;

else if(h_cnt>=H_CNT)

h_cnt <= 11'd0;

else

h_cnt <= h_cnt + 11'd1;

end

always@(posedge CLK)

begin

if(RESET)

v_cnt <= 11'd0;

else if(h_cnt==H_CNT)

begin

if(v_cnt>=V_CNT)

v_cnt <= 11'd0;

else

v_cnt <= v_cnt + 11'd1;

end

end

always@(posedge CLK)

begin

if(RESET)

h_de <= 1'b0;

else if((h_cnt>=296)&&(h_cnt<=1319))

h_de <= 1'b1;

else

h_de <= 1'b0;

end

always@(posedge CLK)

begin

if(RESET)

v_de <= 1'b0;

else if((v_cnt>=35)&&(v_cnt<=802))

v_de <= 1'b1;

else

v_de <= 1'b0;

end

always@(posedge CLK)

begin

if(RESET)

VGA_HS <= 1'b1;

else if(h_cnt<=11'd135)

VGA_HS <= 1'b0;

else

VGA_HS <= 1'b1;

end

always@(posedge CLK)

begin

if(RESET)

VGA_VS <= 1'b1;

else if(v_cnt<=11'd5)

VGA_VS <= 1'b0;

else

VGA_VS <= 1'b1;

end

always@(posedge CLK)

begin

if(h_de == 1'h0)

xCount <= 11'h0;

else

xCount <= xCount + 11'h1;

end

always@(negedge h_de)

begin

if(v_de == 1'h0)

yCount <= 11'h0;

else

yCount <= yCount + 11'h1;

end

assign displayArea = (v_de==1 && h_de==1);

endmodule

最后设计游戏的基本功能

module Play(

input CLK, RESET,

input UP, LEFT, DOWN, RIGHT,

output reg [3:0] VGA_R, VGA_G, VGA_B,

output VGA_HS, VGA_VS

);

parameter SCREEN_H = 1024;

parameter SCREEN_V = 768;

parameter UNIT = 32;

parameter SNAKE_MAX_SIZE = 20;

wire VGA_clk, locked; //65 MHz

wire displayArea;

wire [10:0] xCount, yCount; //x ,y pixel

reg [3:0] direction;

reg [10:0] snakeX[0:SNAKE_MAX_SIZE];

reg [10:0] snakeY[0:SNAKE_MAX_SIZE];

reg [SNAKE_MAX_SIZE:0] snakeBody;

reg [7:0] snake_size;

reg game_over;

reg apple, border;

reg [10:0] appleX = 64;

reg [10:0] appleY = 64;

wire [10:0]rand_X;

wire [10:0]rand_Y;

reg update;

wire cel;

wire in_space;

wire cel_border;

wire out_space;

reg [27:0] count, max_count;

// generate VGA clock (65 MHz) from input clock (100 MHz)

clk_wiz_0 clk_wiz_0(

.clk_in1 (CLK),

.reset (RESET),

.clk_out1 (VGA_clk),

.locked (locked)

);

//VGA controller

VGA_gen VGA_gen(

.CLK(VGA_clk), .RESET(~locked),

.displayArea(displayArea),

.xCount(xCount),.yCount(yCount),

.VGA_HS(VGA_HS),.VGA_VS(VGA_VS)

);

//get random number to place the apple

randomGrid randomGrid(

.VGA_clk(VGA_clk),

.max_X(SCREEN_H-UNIT), .min_X(UNIT),

.max_Y(SCREEN_V-UNIT), .min_Y(UNIT),

.rand_X(rand_X),

.rand_Y(rand_Y)

);

//color cels

cel_color cel_color(

.VGA_clk(VGA_clk),

.displayArea(displayArea),

.xCount(xCount), .yCount(yCount),

.cel(cel), .in_space(in_space), .cel_border(cel_border), .out_space(out_space)

);

integer i,j;

always@(posedge VGA_clk or posedge RESET)

begin

if(RESET)

begin

// place the snake head at display center

i = SCREEN_H/2 - UNIT/2;

j = SCREEN_V/2 - UNIT/2;

snakeX[0] <= (i - i%UNIT);

snakeY[0] <= (j - j%UNIT);

// place apple

i = rand_X;

j = rand_Y;

appleX <= (i - i%UNIT);

appleY <= (j - j%UNIT);

for(i = 1; i < SNAKE_MAX_SIZE; i = i + 1)

begin

// place the invisible snake parts outside the scanning area

snakeX[i] <= SCREEN_H;

snakeY[i] <= SCREEN_V;

end

max_count <= 28'd65_000_000;

snake_size <= 1;

game_over <= 0;

end

else if(~game_over)

begin

if(update)

begin

for(i = 1; i < snake_size; i = i + 1)

begin

snakeX[i] <= snakeX[i - 1];

snakeY[i] <= snakeY[i - 1];

end

case(direction)

4'b1: snakeY[0] <= (snakeY[0] - UNIT);

4'b10: snakeX[0] <= (snakeX[0] - UNIT);

4'b100: snakeY[0] <= (snakeY[0] + UNIT);

4'b1000: snakeX[0] <= (snakeX[0] + UNIT);

endcase

end

else

begin

// Detect if snake head hit the apple

if (apple && snakeBody[0])

begin

i = rand_X;

j = rand_Y;

appleX <= (i - i%UNIT);

appleY <= (j - j%UNIT);

if(snake_size < SNAKE_MAX_SIZE )

snake_size <= snake_size + 1;

end

// Detect if snake head hit border

else if (border && snakeBody[0])

game_over <= 1'b1;

// Detect if snake head hit the snake body

else if (|snakeBody[SNAKE_MAX_SIZE - 1 :1] && snakeBody[0])

game_over <= 1'b1;

end

end

end

//check update

always@(posedge VGA_clk or posedge RESET)

begin

if(RESET)

begin

count <= 28'd0;

update <= 0;

end

else if(count >= max_count)

begin

count <= 28'd0;

update <= 1;

end

else

begin

count <= count + 28'd1;

update <= 0;

end

end

//color cel

always@(posedge VGA_clk)

begin

if(displayArea)

begin

if(border)

begin

if(cel)             {VGA_R, VGA_G, VGA_B} <= 12'h3;

else if(in_space)   {VGA_R, VGA_G, VGA_B} <= 12'h0;

else if(cel_border) {VGA_R, VGA_G, VGA_B} <= 12'h3;

else if(out_space)  {VGA_R, VGA_G, VGA_B} <= 12'h0;

else                {VGA_R, VGA_G, VGA_B} <= 12'h0;

end

else if(|snakeBody)

begin

if(cel)             {VGA_R, VGA_G, VGA_B} <= 12'h0;

else if(in_space)   {VGA_R, VGA_G, VGA_B} <= 12'hfff;

else if(cel_border) {VGA_R, VGA_G, VGA_B} <= 12'h0;

else if(out_space)  {VGA_R, VGA_G, VGA_B} <= 12'hfff;

else                {VGA_R, VGA_G, VGA_B} <= 12'h0;

end

else if(apple)

begin

if(cel)             {VGA_R, VGA_G, VGA_B} <= 12'h700;

else if(in_space)   {VGA_R, VGA_G, VGA_B} <= 12'h0;

else if(cel_border) {VGA_R, VGA_G, VGA_B} <= 12'h700;

else if(out_space)  {VGA_R, VGA_G, VGA_B} <= 12'h0;

else                {VGA_R, VGA_G, VGA_B} <= 12'h0;

end

else

begin

if(cel)             {VGA_R, VGA_G, VGA_B} <= 12'h340;

else if(in_space)   {VGA_R, VGA_G, VGA_B} <= 12'h3;

else if(cel_border) {VGA_R, VGA_G, VGA_B} <= 12'h340;

else if(out_space)  {VGA_R, VGA_G, VGA_B} <= 12'h70;

else                {VGA_R, VGA_G, VGA_B} <= 12'h0;

end

end

end

//init direction

always@(posedge CLK)

begin

if(RESET)

direction  <= 4'b0;

else if(UP)

direction  <= 4'b1;

else if(LEFT)

direction  <= 4'b10;

else if(DOWN)

direction  <= 4'b100;

else if(RIGHT)

direction  <= 4'b1000;

else

direction <= direction;

end

// Detect if the VGA scanning is hitting the border

always @(posedge VGA_clk)

begin

border <= ((xCount <= UNIT) || (xCount > SCREEN_H - UNIT) || (yCount <= UNIT) || (yCount > (SCREEN_V - UNIT)));

end

// Detect if the VGA scanning is hitting the apple

always @(posedge VGA_clk)

begin

apple <= ((xCount >= appleX) & (yCount >= appleY) & (xCount < appleX + UNIT) & (yCount < appleY + UNIT));

end

// Detect if the VGA scanning is hitting the snake head or snake body

always@(posedge VGA_clk)

begin

for(i = 0; i < SNAKE_MAX_SIZE; i = i + 1)

snakeBody[i] <= ((xCount >= snakeX[i]) & (yCount >= snakeY[i]) & (xCount < snakeX[i] + UNIT) & (yCount < snakeY[i] + UNIT));

end

endmodule

结果

XDC文件

FPGA的上下左右按键用来控制蛇的行动。

【总结与思考】

该实验花的大部分时间在了设计完美的图像，所以有些的功能只是做出了最基础的比如蛇吃一个目标后增长，撞到墙游戏暂停，等。